CME396 – PROCESS PLANNING AND COST ESTIMATION

Multiple Choice Questions – Units 1 to 4 (10 per unit, 1 mark each)

Unit 1 – Introduction to Process Planning

1. Process planning acts as the link between

(a) Finance and personnel

(b) Sales and marketing

(c) Purchase and stores

(d) Design and manufacturing

2. In variant (retrieval) CAPP, the process plan for a new part is prepared by

(a) Trial and error on the shop floor

(b) Modifying the standard plan of a similar part family

(c) Creating the plan from scratch using decision logic

(d) Copying the plan of any random part

3. Generative CAPP creates a process plan

(a) Only by editing an existing plan

(b) Without any computer

(c) From scratch using part data and decision logic

(d) Only for assembly operations

4. Group technology arranges similar parts into

(a) Departments

(b) Bills of materials

(c) Part families

(d) Cost centres

5. Which coding system is widely used for part classification in CAPP?

(a) Gantt code

(b) ISO 9001

(c) Opitz code

(d) Taylor code

6. Which of the following is NOT an objective of process planning?

(a) Selecting the processes

(b) Deciding the sequence of operations

(c) Fixing the salary of workers

(d) Selecting machines and tools

7. Manual process planning is most suitable for

(a) Small shops with low volume and few part types

(b) Fully automated factories

(c) Continuous process industries

(d) Mass production of thousands of part types

8. On an engineering drawing, the surface finish (roughness) is usually specified by the value of

(a) Hardness in HRC

(b) Tolerance in degrees

(c) Density in g/cm³

(d) Ra in micrometres

9. Material selection for a component is mainly based on

(a) Weight of the drawing sheet

(b) Name of the supplier only

(c) Function, properties, processability and cost

(d) Colour of the material only

10. The process planning document that shows the sequence of departments / machines a part passes through is the

(a) Bill of materials

(b) Route sheet

(c) Purchase order

(d) Invoice

Unit 2 – Process Planning Activities

1. Cutting speed in turning is given by (D in mm, N in rpm)

(a) V = DN / π m/min

(b) V = πDN / 1000 m/min

(c) V = 1000 / πDN m/min

(d) V = πD / N m/min

2. A device that holds and locates the workpiece and also guides the cutting tool is called a

(a) Jig

(b) Mandrel

(c) Fixture

(d) Vice

3. A device that holds and locates the workpiece but does NOT guide the tool is called a

(a) Template

(b) Fixture

(c) Jig

(d) Drill bush

4. The 3-2-1 principle is used in jigs and fixtures for

(a) Coolant supply

(b) Clamping force calculation

(c) Tool sharpening

(d) Location of the workpiece

5. How many locating points are used in the 3-2-1 principle?

(a) 3

(b) 12

(c) 9

(d) 6

6. Break-even point is the level of production at which

(a) Total revenue equals total cost

(b) Variable cost is zero

(c) Fixed cost is zero

(d) Profit is maximum

7. Break-even quantity is given by

(a) Selling price ÷ Fixed cost

(b) Fixed cost ÷ (Selling price per unit − Variable cost per unit)

(c) Variable cost ÷ Fixed cost

(d) (Fixed cost + Variable cost) ÷ Selling price

8. The quality tool that plots process measurements against upper and lower control limits is the

(a) Route sheet

(b) Gantt chart

(c) Bill of materials

(d) Control chart

9. The operation sheet gives details of

(a) Only the delivery date

(b) Only the selling price

(c) Each operation – machine, tools, speed, feed and time

(d) Only the raw material cost

10. Increasing the feed in turning (other conditions same) will

(a) Have no effect on time

(b) Increase machining time and improve finish

(c) Reduce tool wear to zero

(d) Reduce machining time but increase surface roughness

Unit 3 – Introduction to Cost Estimation

1. Estimation of the cost of a product is done

(a) Before production starts

(b) Only after production is completed

(c) Only by the cost accountant

(d) After the product is sold

2. Prime cost is equal to

(a) Factory cost + Office overheads

(b) Direct material + Direct labour + Direct expenses

(c) Direct material + Overheads

(d) Total cost + Profit

3. Factory cost is equal to

(a) Total cost − Selling overheads

(b) Direct material + Direct labour

(c) Prime cost + Factory overheads

(d) Prime cost + Profit

4. Salary of the office staff is an example of

(a) Administrative overhead

(b) Factory overhead

(c) Direct labour

(d) Selling overhead

5. Cost of lubricating oil and cotton waste used in a machine shop is

(a) Indirect material

(b) Direct material

(c) Selling overhead

(d) Direct expense

6. A firm produces 600 machines a year with total overheads of Rs. 1,80,000. Overhead per machine by unit rate method is

(a) Rs. 180

(b) Rs. 3,000

(c) Rs. 600

(d) Rs. 300

7. The most accurate method of allocating overheads in a machine shop is the

(a) Unit rate method

(b) Percentage of direct material method

(c) Machine hour rate method

(d) Percentage of prime cost method

8. In the straight line method, the annual depreciation is

(a) C × n

(b) (C + S) / n

(c) (C − S) / n

(d) S / (C × n)

9. Depreciation charged is highest in the early years of the asset's life in the

(a) Unit rate method

(b) Straight line method

(c) Machine hour method

(d) Reducing balance method

10. The costing method suitable for cement, sugar and chemical industries is

(a) Process costing

(b) Operating costing

(c) Contract costing

(d) Job costing

Unit 4 – Production Cost Estimation (Forging, Welding, Foundry)

1. In forging, the metal lost as iron oxide when the stock is heated is called

(a) Sprue loss

(b) Flash loss

(c) Scale loss

(d) Tonghold loss

2. Scale loss in forging is generally taken as about

(a) 0.5% of net weight

(b) 50% of net weight

(c) 25% of net weight

(d) 6% of net weight

3. The excess metal squeezed out between the die halves in closed-die forging is called

(a) Scale

(b) Sprue

(c) Burr

(d) Flash

4. Gross weight of a forging is equal to

(a) Losses only

(b) Net weight × Density

(c) Net weight − Losses

(d) Net weight + Losses

5. The pattern allowance provided to compensate for contraction of metal on cooling is

(a) Shake allowance

(b) Shrinkage allowance

(c) Draft allowance

(d) Machining allowance

6. The taper given on the vertical faces of a pattern for easy withdrawal from the mould is

(a) Distortion allowance

(b) Shrinkage allowance

(c) Draft allowance

(d) Machining allowance

7. Cross-sectional area of a single-V butt weld (thickness t, root gap g, included angle θ, no reinforcement) is

(a) g t + t² tan(θ/2)

(b) g t + t² sin θ

(c) g + t tan θ

(d) ½ t²

8. Operator factor in welding is the ratio of

(a) Arc time to total working time

(b) Total time to arc time

(c) Electrode used to weld deposited

(d) Voltage to current

9. In foundry cost estimation, gates, runners and risers are

(a) Treated as direct labour

(b) Ignored completely

(c) Remelted as returns and credited at scrap value

(d) Added to the finished casting weight

10. In soldering, the filler metal melts at a temperature

(a) Equal to the melting point of the base metal

(b) Above the melting point of steel

(c) Above 1500 °C

(d) Below 450 °C

ANSWER KEY

| Q.No | Unit 1 | Unit 2 | Unit 3 | Unit 4 |
| --- | --- | --- | --- | --- |
| 1 | (d) | (b) | (a) | (c) |
| 2 | (b) | (a) | (b) | (d) |
| 3 | (c) | (b) | (c) | (d) |
| 4 | (c) | (d) | (a) | (d) |
| 5 | (c) | (d) | (a) | (b) |
| 6 | (c) | (a) | (d) | (c) |
| 7 | (a) | (b) | (c) | (a) |
| 8 | (d) | (d) | (c) | (a) |
| 9 | (c) | (c) | (d) | (c) |
| 10 | (b) | (d) | (a) | (d) |

Unit 1 – Introduction to Process Planning

1. (d) Design and manufacturing

2. (b) Modifying the standard plan of a similar part family

3. (c) From scratch using part data and decision logic

4. (c) Part families

5. (c) Opitz code

6. (c) Fixing the salary of workers

7. (a) Small shops with low volume and few part types

8. (d) Ra in micrometres

9. (c) Function, properties, processability and cost

10. (b) Route sheet

Unit 2 – Process Planning Activities

1. (b) V = πDN / 1000 m/min

2. (a) Jig

3. (b) Fixture

4. (d) Location of the workpiece

5. (d) 6

6. (a) Total revenue equals total cost

7. (b) Fixed cost ÷ (Selling price per unit − Variable cost per unit)

8. (d) Control chart

9. (c) Each operation – machine, tools, speed, feed and time

10. (d) Reduce machining time but increase surface roughness

Unit 3 – Introduction to Cost Estimation

1. (a) Before production starts

2. (b) Direct material + Direct labour + Direct expenses

3. (c) Prime cost + Factory overheads

4. (a) Administrative overhead

5. (a) Indirect material

6. (d) Rs. 300

7. (c) Machine hour rate method

8. (c) (C − S) / n

9. (d) Reducing balance method

10. (a) Process costing

Unit 4 – Production Cost Estimation (Forging, Welding, Foundry)

1. (c) Scale loss

2. (d) 6% of net weight

3. (d) Flash

4. (d) Net weight + Losses

5. (b) Shrinkage allowance

6. (c) Draft allowance

7. (a) g t + t² tan(θ/2)

8. (a) Arc time to total working time

9. (c) Remelted as returns and credited at scrap value

10. (d) Below 450 °C