CME396 – PROCESS PLANNING AND COST ESTIMATION

SECOND CONTINUOUS INTERNAL ASSESSMENT – SET B

ANSWER KEY WITH MARK SPLIT-UP

PART – A (10 × 2 = 20 marks)

1. Why is costing and estimation important in manufacturing?

Answer:

- To fix the selling price and submit quotations / tenders.

- To decide make-or-buy and select the most economical process.

- To prepare budgets, control cost and find the actual profit.

2. List out the various methods of allocation of overhead expenses.

Answer:

- Percentage of direct material cost

- Percentage of direct labour cost

- Percentage of prime cost

- Man-hour rate

- Machine-hour rate

- Combination of man-hour and machine-hour rates

- Unit rate method

3. Define man hour rate and machine hour rate.

Answer:

Man-hour rate = (Total factory overheads) / (Total man-hours worked)

Machine-hour rate = (Total cost of running the machine) / (Total machine hours)

Man-hour rate is used for manual work; machine-hour rate where work is done mainly on machines.

4. Brief the term administrative expenses.

Answer:

Administrative expenses are the costs of running the office (not the shop floor) – salaries of manager and office staff, office rent, stationery, telephone, postage, legal and audit fees. They are added to factory cost to give production (office) cost.

5. List the different losses incurred in a forging shop.

Answer:

- Shear (cutting) loss

- Scale loss

- Flash loss

- Tonghold loss

- Sprue loss

- Burr (punching) loss

6. Distinguish between welding and soldering.

Answer:

- Welding – base metals are melted and fused; high temperature (above 1500 °C for steel); strong, permanent joint.

- Soldering – base metals are not melted, only the filler (solder) melts below 450 °C; weak joint used for electrical and sealing work.

7. What is tonghold loss and scale loss in forging?

Answer:

- Tonghold loss – extra length of bar held by the tongs during forging and cut off later (about 1 to 1.5 × bar diameter).

- Scale loss – metal lost as iron oxide scale formed on heating; about 6% of net weight for the first heating.

8. What are the factors to be considered when estimating jobs in a forging shop?

Answer:

- Net weight of forging and the losses (scale, flash, tonghold, shear, sprue) → gross weight and stock size.

- Cost of material, heating (fuel / power), forging labour and dies.

- Number of heats, type of forging (hand / drop / press) and overheads.

9. Why is machining time calculation important in manufacturing processes?

Answer:

- To estimate labour cost and machine-hour cost of a job.

- To fix delivery dates, plan production and load machines.

- To fix incentive (bonus) rates and check worker efficiency.

10. Write down the formulae for calculating machining time for milling operations.

Answer:

fm = ft × Z × N mm/min,     T = (L + A + O) / fm min

ft = feed per tooth, Z = number of teeth, N = cutter rpm, L = length of job, O = over-travel. Approach for slab milling A = √(d (D − d)); for face milling (cutter centred) A = D/2.

PART – B (5 × 13 = 65 marks)

11.1

Explain the elements of cost and the ladder of cost with a neat sketch.

Split-up (13 Marks)

- Elements of cost – material, labour, expenses (direct and indirect) with examples → 5 marks

- Definition and classification of overheads → 1 mark

- Ladder of cost relations – prime, factory, production, total cost and selling price → 3 marks

- Neat sketch of ladder of cost → 4 marks

11.2

Explain the steps involved in estimating labour and material costs in manufacturing.

Split-up (13 Marks)

- Material cost – volume from drawing, weight = volume × density, allowances, cost per kg, scrap value → 5 marks

- Labour cost – operations, set-up / operation / handling time, allowances, time per piece formula, labour cost = time × rate → 5 marks

- Illustrative numerical example → 3 marks

(Either 11.1 or 11.2 to be answered.)

12.1

Discuss the various methods of costing, such as job costing, process costing and activity-based costing, highlighting their differences and suitability for different types of manufacturing processes.

Split-up (13 Marks)

- Explanation of job, batch, process, unit, operating, contract and activity-based costing → 7 marks

- Comparison table – job vs process vs ABC → 4 marks

- Suitability and conclusion → 2 marks

12.2

Elaborate the concept of appreciation and depreciation. Explain the various methods of calculating depreciation with an example.

Split-up (13 Marks)

- Definition and causes of depreciation → 2 marks

- Concept of appreciation with example → 2 marks

- Methods with formulae – straight line, reducing balance, sinking fund, sum-of-years' digits, machine-hour → 6 marks

- Numerical example using the methods → 3 marks

(Either 12.1 or 12.2 to be answered.)

13.1

Explain the steps involved in estimating a job in a forging shop.

Split-up (13 Marks)

- Steps – net weight, losses, gross weight, stock size, material, labour, die and heating cost, overheads → 6 marks

- Formulae – gross weight, tonghold, flash volume, stock length → 3 marks

- Worked example (net volume, losses, gross weight, stock length, material cost) → 4 marks

13.2

Illustrate the procedure for doing real time estimation in a modernized semi-automated foundry shop with a case example.

Split-up (13 Marks)

- Meaning of real-time estimation (sensors, ERP, live data) → 2 marks

- Procedure – CAD weight, gating simulation, metal charge, melting energy, moulding cycle cost, pattern cost, fettling / inspection, overheads → 6 marks

- Case example with cost table per casting → 4 marks

- Conclusion → 1 mark

(Either 13.1 or 13.2 to be answered.)

14.1

Discuss the specific challenges and considerations involved in estimating production costs for different types of jobs in forging, welding and foundry shops.

Split-up (13 Marks)

- Forging – challenges and considerations → 3 marks

- Welding – challenges and considerations → 3 marks

- Foundry – challenges and considerations → 3 marks

- Common challenges → 2 marks

- Remedies / ways to improve accuracy → 2 marks

14.2

Two MS plates 1 m long, 10 mm thick, single V butt joint (60°, root gap 2 mm). Density 7.8 g/cm³; deposition efficiency 70%; electrode Rs. 120/kg; welding speed 0.15 m/min; operator factor 50%; labour Rs. 200/h; overheads 100% of labour; 25 V, 200 A, η = 60%; power Rs. 8/kWh. Estimate the total cost of welding.

Split-up (13 Marks)

- Cross-section A = g t + t² tan(θ/2) = 77.74 mm² → 2 marks

- Weight of weld metal = 0.606 kg → 2 marks

- Electrode required 0.866 kg; electrode cost Rs. 103.94 → 2 marks

- Arc time 6.67 min, total time 13.33 min; labour Rs. 44.44; overheads Rs. 44.44 → 3 marks

- Power = (V I / 1000η) × arc time = 0.926 kWh → Rs. 7.41 → 2 marks

- Total cost table and final answer → 2 marks

Key answer:

Total cost of welding ≈ Rs. 200.23.

(Either 14.1 or 14.2 to be answered.)

15.1

Illustrate the steps in finding the machining time for a sample component which consists of: (i) Drilling and tapping (6) (ii) Boring and taper turning (7).

Split-up (13 Marks)

(i) Drilling and tapping (6 marks)

- Drilling – N, L = l + 0.3d, T = L/(f N) with sample values → 3 marks

- Tapping – feed = pitch, T = 2L/(p N) with sample values → 3 marks

(ii) Boring and taper turning (7 marks)

- Boring – number of cuts, N, time → 3 marks

- Taper turning – taper length, number of cuts, N at mean diameter, time → 4 marks

15.2

Explain the steps in calculating the machining time for shaping and planing operations with an illustrative numerical case example.

Split-up (13 Marks)

- Steps and formulae – stroke length, width, number of strokes, K, time per stroke, total time → 5 marks

- Difference between shaper and planer working → 2 marks

- Numerical example(s) with answer → 6 marks

(Either 15.1 or 15.2 to be answered.)

PART – C (1 × 15 = 15 marks)

16.1

Raw material Ø45 × 95 mm: face both ends to 90 mm; turn Ø40 full length; turn Ø30 over 70 mm and Ø25 over 45 mm; 45° × 3.5 mm chamfer; thread (pitch 3 mm) over 35 mm; drill Ø15 × 20 mm. Vturn = 30 m/min, f = 0.35 mm/rev, depth 1.25 mm; Vdrill = 30 m/min, f = 0.1 mm/rev; chamfer feed 0.25 mm/rev; Vthread = 10 m/min. Find the total machining time.

Split-up (15 Marks)

- Facing both ends (2 cuts per end) → 1.212 min → 3 marks

- Turning Ø45 → Ø40 (2.424 min), Ø40 → Ø30 (3.351 min), Ø30 → Ø25 (0.808 min) → 6 marks

- Chamfering → 0.037 min → 1 mark

- Drilling, L = l + 0.3d → 0.385 min → 2 marks

- Thread cutting, 10 cuts → 0.916 min → 2 marks

- Summary table and total time → 1 mark

Key answer:

Total machining time ≈ 9.13 min per piece (approach and over-travel neglected; thread depth per cut taken as 0.2 mm).

16.2

Discuss the significance of machining time calculation. How is machining time calculated for milling, shaping and grinding operations? Explain with suitable formulae.

Split-up (15 Marks)

- Significance of machining time calculation → 3 marks

- Milling – table feed, approach, time formula with example → 4 marks

- Shaping – strokes, K, time formula with example → 4 marks

- Grinding – traverse feed, passes, time formula with example → 3 marks

- Conclusion → 1 mark

(Either 16.1 or 16.2 to be answered.)