|  | ST. JOSEPH’S COLLEGE OF ENGINEERING AND TECHNOLOGY A.S Nagar, Elupatti– Thanjavur. (Approved by AICTE-New Delhi, Affiliated to Anna University, Chennai.) |
| --- | --- |
|  | Department of Mechanical Engineering |
|  | CONTINUOUS INTERNAL ASSESSMENT TEST-II, OCT - 2026 |

| Reg.No | 8 | 2 | 1 | 9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Regulation | R2021 |  |  |  |  |  |  |  |  |  |  |  |  |
| Course Code-Name | CME396 & Process Planning and Cost Estimation |  |  |  |  |  |  |  |  | SET- B |  |  |  |
| Year / Semester | III & V |  |  |  |  |  |  |  |  | Total Marks:100 |  |  |  |
| Date - Time | __.10.2026 / 9.30AM TO 12.30PM |  |  |  |  |  |  |  |  |  |  |  |  |
| Course Instructor | Mr. C.Anand Chairman |  |  |  |  |  |  |  |  |  |  |  |  |
| C303PEIIIV10.4.3-To explain the concept of cost estimation. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C303PEIIIV10.4.4-To compute the job order cost for different types of shop floor. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| C303PEIIIV10.4.5-To calculate the machining time for various machining operations. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L1-Remember , L2-Understand , L3-Apply , L4-Analyze , L5 -Evaluate , L6-Create |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Answer All Questions Part - A (10*2=20 Marks) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Q.NO | QUESTIONS | Marks | CO | LEVEL |
| 1. | Why is costing and estimation important in manufacturing? | 2 | C303PEIIIV10.4.3 | U |
| 2. | List out the various methods of allocation of overhead expenses. | 2 | C303PEIIIV10.4.3 | R |
| 3. | Define man hour rate and machine hour rate. | 2 | C303PEIIIV10.4.3 | R |
| 4. | Brief the term administrative expenses. | 2 | C303PEIIIV10.4.3 | U |
| 5. | List the different losses incurred in a forging shop. | 2 | C303PEIIIV10.4.4 | R |
| 6. | Distinguish between welding and soldering. | 2 | C303PEIIIV10.4.4 | U |
| 7. | What is tonghold loss and scale loss in forging? | 2 | C303PEIIIV10.4.4 | R |
| 8. | What are the factors to be considered when estimating jobs in a forging shop? | 2 | C303PEIIIV10.4.4 | U |
| 9. | Why is machining time calculation important in manufacturing processes? | 2 | C303PEIIIV10.4.5 | U |
| 10. | Write down the formulae for calculating machining time for milling operations. | 2 | C303PEIIIV10.4.5 | R |
| Part - B (5*13= 65 Marks) |  |  |  |  |
| 11.a | Explain the elements of cost and the ladder of cost with a neat sketch. | 13 | C303PEIIIV10.4.3 | Ap |
| (OR) |  |  |  |  |
| 11.b | Explain the steps involved in estimating labour and material costs in manufacturing. | 13 | C303PEIIIV10.4.3 | Ap |
| 12.a | Discuss the various methods of costing, such as job costing, process costing and activity-based costing, highlighting their differences and suitability for different types of manufacturing processes. | 13 | C303PEIIIV10.4.3 | Az |
| (OR) |  |  |  |  |
| 12.b | Elaborate the concept of appreciation and depreciation. Explain the various methods of calculating depreciation with an example. | 13 | C303PEIIIV10.4.3 | Ap |
| 13.a | Explain the steps involved in estimating a job in a forging shop. | 13 | C303PEIIIV10.4.4 | Ap |
| (OR) |  |  |  |  |
| 13.b | Illustrate the procedure for doing real time estimation in a modernized semi-automated foundry shop with a case example. | 13 | C303PEIIIV10.4.4 | Ap |
| 14.a | Discuss the specific challenges and considerations involved in estimating production costs for different types of jobs in forging, welding and foundry shops. | 13 | C303PEIIIV10.4.4 | Az |
| (OR) |  |  |  |  |
| 14.b | Two mild steel plates of 1 m length and 10 mm thickness are to be joined by a single V butt joint (included angle 60°, root gap 2 mm, reinforcement neglected) using arc welding. Estimate the total cost of welding with the following data: Density of steel = 7.8 g/cm³; electrode deposition efficiency = 70%; cost of electrode = Rs. 120 per kg; welding (arc) speed = 0.15 m/min; operator factor = 50%; labour rate = Rs. 200 per hour; overheads = 100% of labour cost; welding voltage = 25 V, current = 200 A, efficiency of the welding machine = 60%; cost of power = Rs. 8 per kWh. | 13 | C303PEIIIV10.4.4 | Ap |
| 15.a | Illustrate the steps in finding the machining time for a sample component which consists of the following operations: (i) Drilling and tapping. (6) (ii) Boring and taper turning. (7) | 13 | C303PEIIIV10.4.5 | Ap |
| (OR) |  |  |  |  |
| 15.b | Explain the steps in calculating the machining time for shaping and planing operations with an illustrative numerical case example. | 13 | C303PEIIIV10.4.5 | Ap |
| Part - C (1*15= 15 Marks) |  |  |  |  |
| 16.a | A component is to be made from a 45 mm diameter and 95 mm long raw material. Both ends are faced to a final length of 90 mm. The component is then turned to 40 mm diameter over its full length, turned to 30 mm diameter over a length of 70 mm and to 25 mm diameter over a length of 45 mm from the same end. The 25 mm end has a 45° × 3.5 mm chamfer and is threaded over 35 mm length with 3 mm pitch. A central hole of 15 mm diameter × 20 mm depth is drilled at the other end. Find the total machining time with the following data: Cutting speed for turning = 30 m/min; feed for turning = 0.35 mm/rev; depth of cut = 1.25 mm; cutting speed for drilling = 30 m/min; feed for drilling = 0.1 mm/rev; feed for chamfering = 0.25 mm/rev; cutting speed for thread cutting = 10 m/min. | 15 | C303PEIIIV10.4.5 | Ap |
| (OR) |  |  |  |  |
| 16.b | Discuss the significance of machining time calculation. How is machining time calculated for milling, shaping and grinding operations? Explain with suitable formulae. | 15 | C303PEIIIV10.4.5 | Ap |

Course Instructor	HOD	IQAC Coordinator