International Conference on Advances in Aerospace and Navigation Systems (ICAAN–2026)

Track: Artificial Intelligence in Aerospace

Machine Learning Surrogate Model for Rapid Prediction of Lift and Suction Peak of NACA 4-Digit Airfoils

Dr. S. R. Sathishkumar1*, [Staff Name]1, [Student 1]2, [Student 2]2, [Student 3]2, [Student 4]2

¹Assistant Professor, ²UG Student, Department of Mechanical Engineering,

St. Joseph’s College of Engineering and Technology, Thanjavur – 613 403, Tamil Nadu, India

*Corresponding author: [email address]

Abstract

Aerodynamic analysis of airfoils during the preliminary design of aircraft and unmanned aerial vehicles (UAVs) requires repeated flow computations, which become time-consuming when many shape variants must be evaluated. This work presents a machine learning (ML) surrogate model that instantly predicts the lift coefficient (CL) and the minimum pressure coefficient (Cp,min, suction peak) of NACA 4-digit airfoils. A Hess–Smith source–vortex panel method with 160 panels was implemented in Python and validated against thin airfoil theory: the computed lift-curve slope of NACA 0012 was 0.120 per degree, and the zero-lift angle of NACA 2412 was −2.13°, compared with −2.08° predicted by theory. Using the validated solver, a dataset of 3,000 cases was generated by randomly varying the maximum camber (0–6%), camber position (20–60% of chord), thickness (8–18%) and angle of attack (−4° to 12°). Linear regression, random forest and artificial neural network (ANN, two hidden layers of 64 neurons) models were trained on 80% of the data and tested on the remaining 20%. The ANN gave the best accuracy, with a coefficient of determination (R²) of 0.999 for both CL and Cp,min and mean absolute errors of 0.016 and 0.042 respectively, whereas linear regression could not capture the nonlinear suction peak (R² = 0.66). Angle of attack and camber were found to dominate the lift. The trained ANN predicts each case about 1,500 times faster than the panel method, enabling rapid design-space exploration of UAV wing sections. The present study is limited to inviscid, incompressible flow; viscous effects and stall will be included in future work.

Keywords: Airfoil; Panel method; Machine learning; Artificial neural network; Surrogate model; UAV